# Migrating Raspberry Pi 4B from Fedora Minimal to Rocky Linux 9 + AdGuard Home

Date: March 28, 2026
Hardware: Raspberry Pi 4B (2GB RAM), external USB SSD
OS: Rocky Linux 9 (ARM64, Minimal)
Skills demonstrated: Linux system administration, SSH hardening, firewall management, DNS configuration, network security

## Why I Did This

My Raspberry Pi was previously running Fedora Minimal with Pi-hole for network-wide ad blocking. I made the decision to migrate away from Fedora for principled reasons related to its direction on age verification compliance — a pattern I recognize as incremental surveillance overreach. Rocky Linux 9 is a 1:1 RHEL-compatible rebuild, which also makes it ideal lab hardware for my RHCSA certification path.

I replaced Pi-hole with AdGuard Home at the same time. AdGuard Home is more actively maintained, has a cleaner web interface, and handles both DNS filtering and DHCP in one package.

## What I Built

A headless Rocky Linux 9 server running on a Raspberry Pi 4B with:

- A non-root admin user with sudo privileges

- Hardened SSH (key-based authentication only, root login disabled)

- firewalld configured for DNS and web interface access

- AdGuard Home running as a system service, owning port 53

- Network-wide DNS filtering for all devices on the local network

## Step 1 — Flash Rocky Linux to External SSD

Rocky Linux 9 ships as a compressed .xz image. I decompressed it first, then wrote it directly to the external SSD using dd.

# Decompress the image

xz -d /home/rokashi/Documents/RockyRpi_9.2.img.xz

# Write to the external SSD (/dev/sda)

sudo dd if=/home/rokashi/Documents/RockyRpi_9.2.img of=/dev/sda bs=4M status=progress

What each flag does:

- if= — input file (the Rocky image)

- of= — output file (the target device, the SSD)

- bs=4M — writes 4MB at a time instead of the tiny default, making the process significantly faster

- status=progress — shows live transfer progress so it doesn't look frozen

Result: 3.5GiB written to /dev/sda in 85 seconds. SSD plugged into the Pi, booted cleanly into Rocky Linux 9 Minimal.

## Step 2 — Create a Non-Root User with Sudo

Rocky Linux (like all RHEL-family systems) grants sudo access through the wheel group. Operating as root directly is a security and accident liability — a named user with sudo is the correct pattern.

# Create the user with a home directory and add to wheel group in one command

sudo useradd -m -G wheel cassian

# Set the password

sudo passwd cassian

Flags:

- -m — creates /home/cassian automatically

- -G wheel — adds the user to the wheel group, which Rocky grants full sudo access by default

Verified with:

id cassian

# Expected output: uid=1000(cassian) gid=1000(cassian) groups=1000(cassian),10(wheel)

## Step 3 — System Update

Before anything else, update all packages on the fresh install.

sudo dnf update

Note: The first attempt failed — the root filesystem ran out of space mid-update (486 packages). I repartitioned to expand the available space, then ran the update again successfully. This is a known issue with the default Rocky Pi image partition layout.

After the update completed:

sudo reboot

## Step 4 — Enable and Harden SSH

### Enable the SSH daemon

sudo systemctl enable --now sshd

enable --now does two things in one command: configures sshd to start at every boot, and starts it immediately without a separate start command.

### Open the firewall for SSH

Rocky uses firewalld by default. SSH needs to be explicitly permitted:

sudo firewall-cmd --permanent --add-service=ssh

sudo firewall-cmd --reload

--permanent makes the rule survive reboots. --reload applies it to the running firewall without restarting the service.

### Harden the SSH configuration

sudo nano /etc/ssh/sshd_config

Changed three settings (uncommented and set):

PermitRootLogin no          # No direct root login over SSH

PasswordAuthentication yes  # Temporarily kept on until keys are set up

PubkeyAuthentication yes    # Enable key-based authentication

sudo systemctl restart sshd

sudo systemctl status sshd  # Confirmed: active (running)

## Step 5 — Set Up SSH Key Authentication

Key-based authentication means no password is ever sent over the network. A keypair is generated on the client (laptop); the public key is placed on the server (Pi). Only someone holding the private key can log in.

### Generate the keypair (on the laptop)

ssh-keygen -t ed25519 -C "rocky-pi"

- ed25519 — modern elliptic curve algorithm, stronger and smaller than RSA

- -C — a label to identify the key purpose

This creates two files:

- ~/.ssh/id_ed25519 — private key, never leaves the laptop

- ~/.ssh/id_ed25519.pub — public key, safe to copy anywhere

### Copy the public key to the Pi

ssh-copy-id cassian@192.168.8.110

Note: This threw a WARNING: REMOTE HOST IDENTIFICATION HAS CHANGED error. This was expected — the Pi's host key changed because it's a completely new OS install at the same IP address. Resolved by removing the stale entry from known_hosts:

ssh-keygen -R 192.168.8.110

Then ran ssh-copy-id again successfully. The public key was written to /home/cassian/.ssh/authorized_keys on the Pi.

### Disable password authentication

With keys confirmed working, went back into sshd_config and set:

PasswordAuthentication no

sudo systemctl restart sshd

Now only key holders can authenticate. Password-based SSH login is closed.

## Step 6 — Install AdGuard Home

### Why install before stopping the system resolver

AdGuard Home takes over port 53 (DNS). The system's built-in resolver (systemd-resolved) also uses port 53 — they cannot run simultaneously. The correct sequence is:

- Download and install AdGuard while the system resolver is still alive (DNS still works)

- Then stop the system resolver and hand port 53 to AdGuard

Installing first means there's no window where DNS is dead and nothing can resolve hostnames.

### Run the install script

curl -s -S -L https://raw.githubusercontent.com/AdguardTeam/AdGuardHome/master/scripts/install.sh | sh -s -- -v

The script automatically detects the architecture (ARM64), downloads the correct binary to /opt/AdGuardHome/, registers it as a systemd service, and starts it. No manual service registration needed.

### Open firewall ports

sudo firewall-cmd --permanent --add-port={53/udp,53/tcp,3000/tcp,80/tcp}

sudo firewall-cmd --reload

| Port | Protocol | Purpose |
| --- | --- | --- |
| 53 | UDP + TCP | DNS queries from all network devices |
| 3000 | TCP | AdGuard Home setup wizard (initial setup only) |
| 80 | TCP | AdGuard Home web interface (post-setup) |

## Step 7 — Configure AdGuard Home

Opened http://192.168.8.110:3000 in a browser and ran the setup wizard:

- DNS listen interface: All interfaces

- DNS port: 53

- Web interface port: 80 (so the dashboard is accessible at http://192.168.8.110 without specifying a port)

- Set admin username and password

### Verify DNS ownership

ss -tulpn | grep -E ':53|:80'

udp   UNCONN 0   0   *:53   *:*

tcp   LISTEN 0   4096   *:80   *:*

tcp   LISTEN 0   4096   *:53   *:*

Port 53 (UDP and TCP) and port 80 confirmed active. AdGuard Home owns DNS.

## Step 8 — Point Devices to the Pi

The ISP router (Wistron NeWeb) does not expose a custom DNS field in its DHCP settings, so network-wide DNS had to be configured per device.

### On the laptop (Fedora, NetworkManager)

sudo nmcli con mod "Team Jesus" ipv4.dns "192.168.8.110" ipv4.ignore-auto-dns yes

sudo nmcli con up "Team Jesus"

- ipv4.dns — sets the Pi as the DNS server for this connection

- ipv4.ignore-auto-dns yes — ignores whatever DNS the router tries to push via DHCP

Verified:

resolvectl status

# Current DNS Server: 192.168.8.110 ✓

AdGuard Home dashboard query log immediately showed DNS requests coming in from the laptop.

## Outcome

| Component | Status |
| --- | --- |
| Rocky Linux 9 Minimal | Running, fully updated |
| Non-root user (wheel) | Active |
| SSH (key-only, no root) | Hardened and active |
| firewalld | Configured |
| AdGuard Home v0.107.73 | Running, owns port 53 |
| DNS filtering | Active for configured devices |

The Pi's static IP (192.168.8.110) was already reserved in the router's DHCP table from the previous Pi-hole installation, so no MAC reservation changes were needed.

## Key Lessons

Sequence matters in DNS transitions. Install the replacement service before killing the existing one. There's no reason to have a window where DNS is completely dead.

The known_hosts error is not an attack. It's SSH doing its job — detecting that the host key changed. When you reinstall an OS, that's expected. Remove the stale entry with ssh-keygen -R <ip> and proceed.

Browser-level DNS settings bypass everything. Brave's built-in DNS-over-HTTPS, if set to anything other than OS default, routes around AdGuard entirely. Always check browser-level settings when troubleshooting DNS filtering.

ss -tulpn is your verification tool. Any time you need to confirm a service owns a port, this command shows you exactly what's listening and where.